L10 	WINDOWS

316	STEEL WINDOWS AND DOORS:

-	Manufacturer and reference: Crittall Windows Ltd, Innervision® FR range

-	Generally manufactured in accordance with BS 6510, W20 section

-	Operation and strength characteristics:  To BS 6375: Part 2

-	Resistance to fire:  Internal single doors, double doors and fixed lights tested in accordance with BS 476: Part 22: 1987 Clause 8, offering 30 minutes Integrity-only FD30 fire rating (Passive fire door test – doors are not to be opened in the event of fire)

-	Construction:  All frames have welded corners and are flat and square within normal	manufacturing dimensional tolerances of +/- 1.5 mm.  Intermediate bars are hot tenon riveted and face-welded. Doors must open into designated fire exposed space, glazing beads must be mounted on non-fire exposed face.

-	Galvanizing:  All components including steel attachments, coupling members and ancillaries:  hot-dipped after manufacture to BS EN ISO 1461

-	Glazing details:  7mm Pyrodur fire-resistant glass as standard.

Optional glass types: 15mm Pyrostop, 14mm Pyrobel, 6mm Pyran S

-    	Glazing beads present no horizontal ledges on which dust and dirt can gather

- Glazing method: bead-glazing of single fire-resistant glass utilising Ammonium Phosphate self-adhesive glazing tapes both sides, hardwood setting blocks and low modulus, neutral cure silicone sealant top-capping both sides

- Draughtproofing: 22mm wide x 1mm thick intumescent seals fitted as standard to all doors

- Ironmongery | accessories: See clause 328

- Finish as delivered: Galvanized and Duralife™ polyester powder coated in a range of colours - See clause 331

325	STEEL WINDOWS AND DOORS:

- Generally, all the foregoing

- Manufacturing undertaken in the Crittall factory by directly employed personnel

- Design, manufacture, and installation carried out under Quality Management Systems certified to BS EN ISO 9001

- Manufacture is undertaken under an Environmental Management System certified to BS EN ISO 140001

328	IRONMONGERY/ACCESSORIES:

- Standard provisions for interior Innervision FR doors include:

Brass butt hinges - finished as frames, 4 per leaf

Pair of lever handles set between 1000-1200mm from the finished floor level

Concealed (when door closed) bolts top and bottom of first closing leaf of double doors

Standard Threshold bar (40x6mm high galvanized steel – finished as frames) may be mounted flush with, or on top of, finished floor surface

- Interior Single or Double Doors (non-lockable) additionally fitted with:

Latch operated by lever handles internally and externally

Brass fire-resistant strike

-	Optional hardware: TS83 fire-resistant Door Closers to fire-exposed face only

Single doors (only) offered with europrofile cylinder mortice lock, with deadlock operated by key to non-fire-exposed face and cylinder thumbturn to fire-exposed-face

Europrofile cylinder escutcheons to both faces in a finish to match lever handles

-	Standard handle/escutcheon finishes:  RTD (Bronze Effect) or Satin Chrome

-	Optional hardware finishes:  Available, consult Crittall

331	FINISH COATING:

-	Type/reference:  Epoxy free Duralife® polyester powder coating using Interpon D36 powder

-	To BS EN ISO 13438

-	Preparation:  Following galvanizing, windows are chemically cleaned and pre-treated to provide a surface to which powder coating will adhere

-	Covering:  Minimum 60 microns on all significant surfaces

-	Colours:  Wide selection available.  See Crittall Duralife® Standard Colour Guide

-	Process:  Coating will be undertaken in the same manufacturing plant as the frame fabrication

470	SEALANTS:

- Joint surfaces are to be clean, grease / dust free and prepared in accordance with the sealant manufacturer’s recommendations

- Glass capping sealant is to be a neutral cure low modulus silicone compliant with BS 5889 Type A 1989

- Frame perimeters sealed to structure using intumescent open-cell polyurethane flexible foam infill and finished with Intumescent silicone triangular-profile seals with a maximum joint size of 6 x 6mm

- Coupling open joints sealed with Intumescent silicone sealant

552	METAL GLAZED SCREENS:

-	Screens/coupled assemblies are available

-	A full design/advice service is available